Name: ……………………………………….

1163 (Colne Valley) Squadron

Academics workbook

Level:	Senior

Topic: Aircraft Handling

Answer book

in conjunction with the

Aircraft Handling power point and

Topic book ACP 34, Aircraft Operation, Volume 3, Aircraft Handling

Contents and sessions:

Session 1	Aircraft structure revision,

Airmanship 1 revision 	Airfield Features, Passenger briefing

Airmanship 2 revision		Air Traffic Control, Rules of the Air

Session 2	Aircraft Maintenance

Session 3	Ground Handling

Session 4	Marshalling

Session 5	Pre-flight briefing

Session 6	General Flying

Session 7	General Flying

Session 8	Aerobatics

Session 9	Formation flying

Session 10	Emergency procedures

Session 11	Review

Session 12	Revision, practice tests

Session 13 	Test

Session 14	Test catch up if needed

Abbreviations to be filled in as learned or revised

Aircraft Structure, revision

1.	Cards of aircraft parts will be dealt out round the room.  You must put your cards onto the correct parts of the aircraft model.

2.	Use the labels below to label this aircraft.

| Cockpit canopy | Port position light, red | Starboard flap |
| --- | --- | --- |
| Elevator | Port wing | Starboard main wheel, landing gear |
| Engine cowling | Port wingtip | Starboard position light, green |
| Engine exhaust | Propeller | Starboard wing |
| Fin | Radio antenna | Starboard wingtip |
| Fuselage | Rear facing position light, white | Tailplane |
| Landing lights | Rotating beacon, red | Trailing edge |
| Nosewheel and nose gear | Rudder | Trim tab |
| Oil cooler | Spinner | Wing leading edge |
| Port flap | Starboard aileron | Wing root |

Airmanship 1 Revision, Runways and Pre-flight Safety Briefing

What are the three main types of runway?	1.	……grass ………………

2.	triangular pattern …				3.	…main instrument …

Put the correct number to each of these parts:

| 15 | Aerodrome Identification | 5 | Hangar |
| --- | --- | --- | --- |
| 14 | Aerodrome Identification Beacon | 8 | Hangar |
| 6 | Aircraft Servicing Platform | 13 | Holding points |
| 12 | Arrester Gear | 4 | Main Instrument Runway |
| 17 | Arrester Gear | 2 | Operational Readiness Platform |
| 3 | Barrier | 10 | Second Runway |
| 11 | Barrier | 9 | Sterile Area |
| 18 | Control Tower | 7 | Terminal Building |
| 1 | Dispersal Hard Standing | 16 | Windsock |

Safety Briefing: Give some of the information expected in a safety briefing before a flight:

1.	Emergency actions including action on the ground	………………………………….

2.	Safety equipment such as parachute, life preservers, safety harness 	………

3.	Aim of exercise, local flying area	…………………………………………………….

4.	Weather	……………………………………………………………………….

Airmanship 2 Revision,

Air Traffic Control  (…ATC…)

For safety in the air there must be some control of movements.

Where are controllers generally found on a military airfield?		Control tower……..

What are the two main types of controller?	1. ground and circuit	  2. approach

What other control unit might be with them?		Bird control unit, Ground radio flight

What other controller might be at the airfield? Runway	  where? caravan to left of touch down end

What is the main form of ground to air communication? Radio telephony……………………..  (RT..)

What are the two main systems?   Very high frequency  (VHF),     Ultra high frequency   (UHF…)

What visual communications are there?	signal lamp, ground markings such as threshold

The aircraft can communicate with a system that transmits and receives.  What is this system?

Radio Detection and Ranging				 generally known as RADAR	………………

There are two types: 1.Surveillance Radar	 and 2. Precision Approach Radar

What two other main radio aids can be used?

1. Digital Resolution Direction Finding     (DRDF.),  		2. Instrument Landing System (ILS.)

What is an airway?	Controlled area of airspace between two places………………………………….

What must the pilot have to use the airway? Instrument rating	………………..

What must the aircraft have to use the airway? Appropriate radio and nav aids.	……….

Rules of the Air

List 4 types of aircraft starting with those having complete right of way to those who must give way to all.

1.  Balloons.  	2.  Gliders	  3. Airships	  4. Powered aircraft other than airships

Two similar aircraft approaching head on, what must they do?  	Move to the right

Two air craft on converging courses, who must give way?		Aircraft on the left	……………..

How is overtaking done? 	On the right	…………………………………..

Aircraft on finals and aircraft on hold waiting to go on the runway, circle who has right of way.

Two aircraft on finals one higher, one lower.  Circle who has right of way.

On the ground: 1 aircraft being towed, 2 aircraft taxying, 3 other vehicles.  Mark right of way with 1, 2, 3.

For night flying what must be used on the aircraft.  List giving colour and position:  Lights

1.  Port, red.  2.  Starboard, green  3. Tail, red  4. Tail white

1.  AIRCRAFT MAINTENANCE

What is meant by the word ‘maintenance’?

Keeping a vehicle, an appliance, in good, safe working order  …………

What sort of maintenance would you give a bicycle?

Regularly: check tyre pressures, oil chain, keep battery for lights topped up, clean off mud etc

Also: check that nuts and screws are tight, tighten up when needed, wheel balancing, saddle height ……

What sort of aircraft maintenance have we seen at Earl’s Colne airfield?

Pre-flight checks for fuel, oil, lights, radio aerials, propeller, wings, tail, fuselage, instruments  …

What sort of aircraft maintenance have we seen at Conningsby?

Detailed check and restoring/replacing of all parts of the Typhoon ……

RAF maintenance organisation supports a wide range of maintenance. List 10 here:

| 1 | Avionics systems | 6 | Aircraft escape systems |
| --- | --- | --- | --- |
| 2 | Survival equipment | 7 | Weapons |
| 3 | Flight simulators | 8 | Synthetic trainers |
| 4 | SAM systems (surface to air missile) | 9 | Communications and control systems |
| 5 | Motor transport | 10 | Training and testing equipment |

Be ready to explain each one of them.

What are the two main types of maintenance?

1. ……Operational objectives			…..   2. …Maintenance objectives		……..

Give examples of operational objectives, no more than 3 words for each one:

1.  ……surprise attack……  		2.  ……intensive flying…  3. …NATO & national commitments

4.  ……contingency plans			5. ……Peace time tasks

Give examples of maintenance objectives, no more than 3 words for each one:

1.  …minimize costs……  2.  ……minimize hazardous faults	…  3. …improve reliability

Preventative Maintenance

Servicing is:	checks on each flight for consumables such as fuel and faults  eg: pre and post flight

Scheduled maintenance is: regular maintenance to minimize faults, keep in good condition  eg 50 hour.

Condition based maintenance is: continuous monitoring and replacing when needed.

Out of phase maintenance is: dealing with maintenance that does not fit the regular cycles

Corrective Maintenance

deals with a fault so the aircraft is serviceable, not scheduled, uses condition monitoring techniques

Contingency Maintenance

Occurs in war/emergency conditions when scheduled maintenance cannot be done

Modifications

Changes to the aircraft such as adding or replacing with a new technology  eg: GPS

Forms, the …MOD.   form …700. series

For aircraft maintenance there are many forms that have to be filled in to show the work done.

For each of the following give some points that the form covers:

Form 701	Fuels, oils, weight, tyre pressures, alignment records, aircraft dimensions

Form 703	Onboard Software log including updates, revisions etc

Form 705	Flight Serving and Fuel Certificates, previous captain, chief Tech, new captain signatures.

Form 725	Flying log and Fatigue Data Sheet.

Form 725A	Details of Air to Air refuelling (AAR)

Maintenance Data System  (…MDS…)

The maintenance system recording is computerised into 6 elements these are:

1.	Faults				…………     	   2.	Defects	………………………

3.	Modifications			………………     4.	Manpower utilizations		…………………

5.	Task achievement…………………………	  6.	Technical costs and logistics

Maintenance visit

If there is a visit to a maintenance workshop use this space to put down main points seen.

Not yet done due to shut down and grounding

2. GROUND HANDLING

Arrivals and departures

How many in the handling team?		…2…………………………..

Here is a list of activities for the handling team for an arrival.  Put them in the correct order:

| Assist crew unstrapping | 8 |  | 1 | Clear FOD |
| --- | --- | --- | --- | --- |
| Clear FOD | 1 |  | 2 | Marshall incoming aircraft |
| Connect servicing equipment | 4 |  | 3 | Insert chocks |
| Engine shut down | 6 |  | 4 | Connect servicing equipment |
| Fit safety devices | 9 |  | 5 | Position fire extinguishers |
| Insert chocks | 3 |  | 6 | Engine shut down |
| Marshall incoming aircraft | 2 |  | 7 | Position steps |
| Position fire extinguishers | 5 |  | 8 | Assist crew unstrapping |
| Position steps | 7 |  | 9 | Fit safety devices |

Marshalling will be looked at in the next session.

Danger Zones

Around any aircraft can be dangerous.  Give 4 danger zones round an aircraft on the ground.

1.	…Engine intake	……     		   2.	Engine exhaust………………………

3.	Propellers	…live………………     	   4.	Helicopter rotors …blade sailing…

Wheels, brakes and fires

Rub your hands together very hard.  What does it feel like? ………warm……………………..

Look at the bicycle disc brake.  See how it works. What are the two parts of the brake?

1.	Disc…………………     			2.	……Pad ……………………………………

What force is making the brake work?	………Friction ………………………………………..

What happens if there is too much of this force?	Gets hot, can make a fire

For dealing with a brake fire circle the three things you would do:

1.  	a) stand in line with the axle   		b) stand downwind from the axle

c) stand upwind from the axle   		d) run away

2.  	a) hose it down with water      		b) use the fire extinguisher     			c) throw a coat over it    			d) blow it out

3.  a) aim extinguisher downwards 0.3 m from wheels   b) get as near as possible with extinguisher

c) spray upwards with some liquid                             d) just wait for the Fire Brigade.

Manhandling and Towing

Why should an aircraft not be taxied into and out of a hangar?

Carbon monoxide fumes, too much power in confined space, difficulty of movement

Which surfaces should not be pushed?	Control surfaces: ailerons, elevators, fin, flaps i.e moveable

What is the minimum number needed to get an aircraft out of a hangar?  5 …………..

Parking an aircraft

In this table is a list of actions for parking an aircraft.  Number them in the correct order 1 to 8.

| 6 | Apply control locks. |
| --- | --- |
| 5 | Check the electrical services, ignition switches and fuel cocks are turned off. |
| 3 | Double chock the wheels – fore and aft. |
| 2 | Ensure that no part of one aircraft overlaps any part of another. |
| 7 | Fit pitot and static vents covers. |
| 8 | Lock canopies and doors, fit canopy, wheel and engine covers and set drip trays. |
| 1 | Park the aircraft facing into wind |
| 4 | Release the brakes. |

Fuel

Give the abbreviations for the 4 types of fuel: AVGAS, AVTUR, AVTAG, AVCAT…

Give 4 ways of refuelling an aircraft:	jerry can, bowsers, ground tank high pressure, hand pump	..

Refuelling precautions:  fill in the spaces

  - Aircraft and pumping equipment must be …earthed………….

  - Correct grades of fuel….. and oil…. are put into the appropriate tanks.

  - Leave air space in oil tanks for expansion…. of oil when heated.

  - Never refuel an aircraft in a hangar……. or with the …engine….. running

  - Always filter…… the fuel

  - No …lighters………… or ……matches…….. around the aircraft

  - Wear……rubber… or ……crepe……. soled shoes.  (provides insulation)

  - Avoid fuel spillage……… - call a ……fire………. tender if one occurs.

  - ………15……..m exclusion zone for working on electrical or RT equipment

  - ………40………..m exclusion zone for aircraft with engines running

  - ……emergency………. escape routes for refuelling vehicles.

  - Place suitable ……fire…….   …extinguishers………. ready for use.

  - Stand only on the approved ……walkways…….. on the aircraft.

  - Replace …filter….. caps and check they are……fitted…….. properly.

  - Enter details of the refuelling/defuelling in MOD Form ……705……...

Loading and overloading

Who is responsible for loading an aircraft? 	Air quartermaster

For the following overloading effects insert:  increases or reduces

| Increases Take off and landing distances | Increases Stalling speed | Reduces Range |
| --- | --- | --- |
| Reduces Rate of climb | Reduces Endurance | Reduces Ceiling height |

Would a multi-engine aircraft be able to maintain flight on engine failure when overloaded? …no…….

3. MARSHALLING

What is marshalling?		Assisting pilot in safe manoeuvring of aircraft on the ground

Why is it needed?		To ensure safety and that the pilot knows where to go

Following is: 	1.   	a set of marshalling commands

2.	a set of marshalling action pictures.

3.	a set of marshalling descriptions  (on separate sheets)

1.	Commands

2.	Marshalling action pictures

For each picture copy the correct command from the list above at the top.

It is suggested that commands are copied in pencil and so ready for correction.

For each picture cut and paste the marshalling description from the attached page to underneath the marshaller.

It is suggested that descriptions are all allocated to their pictures before and pasting is done.

Then check they are correct.

There are no spares for corrections.

……

4	PREPARATION FOR FLIGHT

Who is responsible for an aircraft, including emergencies?			…captain  …

What is an important characteristic of this person?				… Leadership………

What crew position does s/he normally hold? 				  … Pilot …………

6 topics must be covered before a flight.  Summarise these with up to 3 words each in this table:

| 1 Familiar with aircraft, crew | 2 Aim of flight | 3 Personal, crew fitness |
| --- | --- | --- |
| 4 Relevant order books | 5 Clothing, equipment, safety | 6 Planning, navigation, complete |

and the same for Flight Planning:

| 1 Weather current, forecast | 2 ATC, diversions, restrictions | 3 Nav calculations, charts |
| --- | --- | --- |

Who is involved with Self Briefing?	…Captain, navigator ………………………

Who is involved with mass briefing?  	Everybody	……………………………………………………

Who is responsible for Passenger Briefing?   …Captain….  or delegated to: Air Loadmaster

7 topics in passenger briefing.  In the table give a summary for each one with up to 3 words:

| 1 Captain in command | 2 Safety, crash, ditching |  | 3 Oxygen masks, controls |
| --- | --- | --- | --- |
| 4 Escape hatches, dinghy |  | 5 Fitting, operating parachutes |  |
| 6 Smoking, flame rules |  | 7 R/T communication equipment |  |

Which is the Flight Authorisation form?	……… 3562 ………………………………………….

Who must sign the form?	…Officer eg OC……..   and …… Captain …………

What 5 items must be considered before signing?	1. Weather	…  2. Crew experience	…..

3. …Pilot Instrument Flying Rating….    4. … Equipment available	  5. … Facilities at destination

What happens to this form if there is an accident?  …impounded…….  plus form …700….

Which other form must be signed?   … 705……   when? …before and after sortie

Pre-Flight checks are next.  Summarize these in up to 4 words per slot in this table:

| 1 Other aircraft | 2. Obstructions | 3 Taxiway routes |
| --- | --- | --- |
| 4 FOD | 5 Clear to start engines | 6 Ground condition |
| 7 Chocks | 8 Engine starting aids | 9 Ground crew and fire extinguishers |

What is meant by ‘challenge and response?	………One crew member calls out, another does the check and responds.

……………………………………………………………………………………………

External Checks are needed to make sure the aircraft is properly serviced.

For doing these checks what must be on?  … Brakes  ……………..  and off? … Switches

Fill in this table to list the items that must be checked and think of why the checks are needed:

| Fuselage | Wings | Tail plane | Fuel leaks |
| --- | --- | --- | --- |
| Hydraulic fluid leaks | Inspection panel fastenings | Pitot head | Static vent |
| Control locks | Undercarriage locks | Engine covers | Blanking plates |
| Tyres | Wheels | Fin | Ice |

Before start checks:

In, on, down are important words in the before start checks.  To what do they refer?

In:	… Passengers and crew strapped in 		Down:	…Undercarriage …………..

On:	…Fuel…………… and …… other services eg avionics …………  and …brakes……..

After start checks:

Check the gauges for:	  1. oil pressure		  2. temperatures of oil and cylinder head

At higher rpm test:	1. power  		2. magnetos	. 3. variable pitch prop control

What is the difference between jet and piston engines?	Jet engines do not need to warm up

Taxiing

What is the first thing to check when taxiing?	brakes……………………………………

Should taxiing be fast or slow? …slow..  Why? overheating brakes, other aircraft, stresses, emergencies

Nose wheel aircraft have either nose wheel or differential steering.  How is differential steering done?

Using the brakes, left brake turns left, right brake turns right, tail wheel keeps turning, nose wheel needs continuous turn force.

What effect can the wind have on taxiing?		…weather cocking, turning aircraft into wind, need firm hold on controls

What must the pilot look out for when taxiing?	Obstructions, other aircraft

For tail draggers and some single engine aircraft how will the pilot taxi for this?

Yawing, that is side to side

Pre-take off checks

These are the last checks before flying and are very important for safety.

What is their abbreviation?	PTOC  	and another name? 	vital actions

Using the check list provided give the type of aircraft   Robin HR200  and list the checks:

…Trim……	……Carb Heat………  ……Fuel Pump………………Master………	…Flaps

Mixture	Magnetos……………	   …Lights…………	…Pitot…………………  AI, DI,

Altimeter, Radio……Transponder	Harnesses & hatches, Controls………………

Think of what might happen if any check was omitted.

5    GENERAL FLYING

In what aircraft have you flown?	…N.B. this section is your own experience …………………………

Were you a passenger or did you actually fly the aircraft?…………………………………………………

What flying was done?	Circle the manoeuvres done. Underline as well if you did it.

From where have you flown?……………………………………………………………………………………

Take-off,   Circuit,   Landing,   Straight and level,   Left turn,   Right turn,   Steep turn,    Ascending turn

Descending turn, Roll,   Loop,   Other aerobatics,   Practice Forced Landing (PFL),  Instrument work

This topic will be done together with practice on the flight sim.  As it will not be whole group work so you will have to work through this on your own or with a partner.  Use the book and your own experience to help you.  You can also run through the power point if you want to but this has less detail than the book.

Take-off

Before take-off which checks must be carried out?	………Vital checks……………………….

Where are these usually done?	………………………hold……………………………………..

Who is in the caravan at this place?	…………………Runway controller……………………………

What is his/her job?	Check taking off aircraft for defects eg loose parts, leaks

Most runways have take-off in a specific direction.  Circle the correct one:  into wind, tail wind.

What are at least 2 advantages of a head wind for take-off?

greater air speed, greater lift, less aircraft stress, better directional control……………………………….

The length of the runway is an important factor in take-off.  If not long enough then take-off cannot be carried out.  Think of the difference between a farmer’s field 250m long and Wethersfield at 1900m long.

There are plenty of factors that can affect the length needed. Consideration of runway length is an important part of a flight plan.

In this table are a number of factors.  Colour in all those that necessitate a longer take-off run.

| No wind | Fully loaded | Not much load | No flap |
| --- | --- | --- | --- |
| Some flap | Low power | High power | Uphill |
| Down hill | Snow | Wet grass | Mud |
| Smooth hard surface | High temperature | Low air temperature | High altitude |
| Sea level | Pot holes | Dry ground | A lot of flap |

Having done the vital checks at hold and called for clearance then the pilot taxies to the …runway….

What must be straight for nose wheel aircraft? nose wheel.. and tail wheel aircraft? tail wheel.

Take-off procedure

This table has 16 points related to take off going from when you have done pre-flight checks up to climb-out.  First number them in the order you think they ought to be.  When satisfied they are in the right order copy out in the right order in the numbered section of the table.

One item is missing.  What is it? ……Lift off……..  Mark in where it should be.

Landing procedure

| Keep straight using brakes/rudder | 1 | Taxi to hold | For tailwheel nose down, tail up. | 9 | Keep straight using brakes/rudder |
| --- | --- | --- | --- | --- | --- |
| Engine checks | 2 | Vital checks | Check instruments | 10 | Raise nose slightly |
| Pull control back gently | 3 | Get clearance | Straighten nose/tail wheel | 11 | For tailwheel nose down, tail up |
| Full power | 4 | Line up | Vital checks | 12 | Attain flying speed |
| Raise and lock undercarriage | 5 | Straighten nose/tail wheel | Start rolling | 13 | Pull control back gently Lift off |
| Taxi to hold | 6 | Full power | Raise flaps | 14 | Raise and lock undercarriage |
| Attain flying speed | 7 | Engine checks | Raise nose slightly | 15 | Raise flaps |
| Line up | 8 | Start rolling | Get clearance | 16 | Check instruments |

Which is best for landing, head wind, tail wind, no wind? ………head wind…………………….

Is it possible to land with	a) no engine?	……yes..	b) no flaps?	…… yes ………..

Circle which responds more quickly to opening the throttle: 		jet engine;	 piston engine

What are 2 advantages of flaps?	drag to slow aircraft down

Having done the pre-landing checks, see next section on Circuits, you are ready to come in to land. Although the landing procedure is the basic one it does vary with each field and is dependent on aircraft, wind and field conditions.

Look at the following table of actions for landing. First number them in the order you think they ought to be. When satisfied they are in the right order copy out in the right order in the numbered section of the table.

| Hold off/float | 1 | Turn onto finals | Round out | 8 | Close throttle gently |
| --- | --- | --- | --- | --- | --- |
| Adjust flaps as needed | 2 | Call final | Keep in line with runway | 9 | Round out |
| Vacate runway | 3 | Keep in line with runway | Nose/tail wheel down | 10 | Hold off/float |
| Turn onto finals | 4 | Constant suitable speed | Constant suitable speed | 11 | Main wheels touch down |
| Back pressure on control | 5 | Moderate descent rate | Close throttle gently | 12 | Nose/tail wheel down |
| Landing run | 6 | Adjust flaps as needed | Moderate descent rate | 13 | Landing run |
| Call final | 7 | Back pressure on control | Main wheels touch down | 14 | Vacate runway |

Circuit

There are many shapes and sizes of circuits depending on the local environment and habitation.  A standard circuit has take-off, cross wind, downwind, base and finals.  Only take-off and landing apply to all circuits. In mountainous areas the take-off and landing can be at the same point due to mountains in the way.  In military circuits the crosswind and base are often just a gradual turn from the crosswind to downwind and from downwind to finals rather than specific sections.  This can happen with GA as well.

The diagram shows the circuits at Andrewsfield.  This is a GA airfield.

Find Andrewsfield on your chart.	Hint: N51 53.70  E000 26.95

Which airport does the circuit just go into?	Stansted ……and ICAO code:…EGSS…

What direction are the runways?  09, 27……… What is the circuit height? …700’

Circle the circuit direction:	left hand;	right hand

Does the circuit go over any villages?	…no……..

Name 3 visual reference factors:	…black barn, woods, lakes ………………..

Label north, south, east and west on the diagram.  Mark which side is the commercial airport.

For each circuit label: Take off, crosswind, downwind, base, finals, dead side.

Manoeuvring the aircraft

Having reached cruise height then it is advisable to do the following checks: Try and work them out.

Fuel……….	Radio………..	Engine temp and pressure.	Direction	Altitude

These should be re-done at regular intervals during a flight, otherwise you might get into trouble.

When in cruise you use QNH. What is this? pressure at sea level on which instrument? altimeter

What are the 3 planes of movement of the aircraft?	For each plane give the part that does this movement, the type of movement and how you control it:

| Plane | Around which axis | Part | Control |
| --- | --- | --- | --- |
| Pitching | Lateral | Elevator | Control column |
| Rolling | Longitudinal | Ailerons | Control column |
| Yawing | Normal (vertical) | Rudder | Rudder pedals |

Height and speed must also be considered.

Height:  Would you need more or less power to go: up?	…more…    	 down? …less…………

Speed:   What happens to speed with the nose:       up?	…less………    down?  …more……………

Describe how you would:	(hint do not forget to consider power as well as control)

1.	Keep straight and level:   constant power, nose straight ahead, aircraft level, control central

2.	Turn right:		control right, right aileron up, nose level, more power for steep turns,

3.	Turn left:		control left, left aileron up, nose level, more power for steep turns

4.	Ascend:		more power, nose up using control column

5.	Descend:		less power, nose down using control column

6.	Go slower:		nose up, less power

Circuits

Most airfields will have a circuit pattern.  In this way aircraft can land in an orderly manner.  Each airfield has its own circuit and it is important to have found this out, preferably by having a copy of it with you, for landing.  If you are trying to work out the circuit from the air you are likely to get into trouble for such things as flying in the wrong direction at the wrong height at the wrong time and over the wrong bits of ground.

Airfield circuits can be found either from an airfield book or from the internet.  It is a good idea to phone the airfield first to check if there are any changes.

You have been given the Andrewsfield circuit as an example.  This is not standard in that due to villages the legs are longer than some other airfields.  The joining and circuit heights are also rather lower than many airfields.

Look at your chart and work why the joining and circuit heights are low:	Stansted airspace

What are the five legs for a circuit, give in order from take off and explain each one:

1.	Take-off, aircraft leaves runway and ascends

2. 	Cross wind, turn either left or right depending on circuit, often at 500’ while ascending to circuit height.  In some circuits, especially military this is just a curve round from take-off to downwind

3. 	Downwind, straight and level flight parallel with runway. Radio call of downwind.  Downwind checks, see below.  Some circuits also have flaps lowered downwind.

4. 	Base, lower power to descend, lower speed, flaps if required. In some circuits, especially military this is just a curve round from downwind to finals

5.	Finals.  Descending to land, watching airspeed and runway.  More flaps if needed.  Call finals.

What is dead side?		Side opposite to downwind.

Joining a circuit

There are a number of different ways to join a circuit dependent on traffic, environment, habitation.

It is normal to call to announce approach and request joining instructions. The runway and any other conditions are then given.

Standard approach to a circuit: on the …dead…side and then overhead.. the runway at 2000.ft

It is normal to specify where one is at each of these places so that other traffic and control are aware.

Once overhead the aircraft starts coming down to …circuit   height on the crosswind. leg.

It then joins the downwind.. leg and again announces its position.

On the downwind leg the landing checks are done.  These are dependent on the aircraft.  For a training aircraft on avgas the following are common.  See if you can work them out:

Brakes……….,  		Undercarriage…,  		Mixture…….., 		Fuel……,

Flaps……………, 	Instruments,  		Carb heat, 		Harnesses and hatches

At the end of downwind the aircraft is turned onto …base… leg, where the aircraft speed is

lowered…by …raising the nose.   To lower …….the height . the engine revs

must first be …lowered…..  Once flaps are lowered then the …nose…..is lowered until a

suitable approach… speed is reached before turning onto …finals………

Military circuits are usually oval rather than rectangular.

Other circuits

If you have time, use the internet or other resources to find the circuits for a number of different airfield including military ones.

6 AEROBATICS

This work is to be done in conjunction with the flight sim.  Use the text book for your answers.

You will need to have a model aircraft to work with.  Get one from the model room.

Before starting any aerobatic exercise it is important to carry out the HASELL   checks.

Now fill in the words:

H eight……..	 		A irframe…………..			S ecurity………….

Engine Ts & Ps……..	 	Location…………..			Lookout…………….

What might happen if the height were too low?	……………hit the ground………….

Why should flaps and undercarriage be up?		to give better air flow

Describe the effect of a loose seat?			move about, maybe throw pilot around, danger

What might a loose pen on the floor do?		get caught under a pedal making it immobile

What 3 engine properties must be checked?		temperature, pressure, fuel

From what 3 locations must aerobatics be clear?	airfields, built up areas, controlled airspace .

What must be looked out for in the air?		aircraft and clouds

Aerobatics makes use of flying in a) 2D;   b) 3D.	Circle the correct answer.

Which planes of movement are used in aerobatics?		pitching, rolling, yawing

In the book are listed 13 aerobatic manoeuvres.

Work through these using a model aircraft to help you understand what is done and how it is done.

You will also do some of these on the flight sim.

Make sure you fill in your flight sim experience on the flight sim page.

For each manoeuvre list it in the table below when you have completed it successfully with a model.

| 1 | Loop | 8 | Aileron turn |
| --- | --- | --- | --- |
| 2 | Barrel Roll | 9 | Derry Turn |
| 3 | Slow Roll | 10 | Vertical Eight |
| 4 | Stall Turn | 11 | Horizontal Figure Eight |
| 5 | Roll off the top | 12 | Hesitation rolls |
| 6 | Half Roll and Pull through | 13 | Inverted flight |
| 7 | Upward Roll |  |  |

Then answer the questions on the next page.  They are not in the order of the manoeuvers and will require a bit of research, not just copy and paste.

Aerobatics questions

1.	A number of manoeuvres require a shallow dive as a starter.  Why?

to gain sufficient speed ……………………………………

2.	Which is the only manoeuvre using a yawing plane?	stall turn

3.	What plane does the loop use?	pitching………………….

4.	How much height would you expect to lose in a good slow roll?	none …………….

5.	What is the direction of an aileron turn?	downwards……………………….

6.	Is every aerobatic aircraft allowed to do prolonged inverted flight?	no ……..

7.	Give a reason for this answer. not designed for inverted g force . (+g going against gravity, -g going with gravity)

8.	What happens to stall speed with inverted flight?	increased………

9.	Explain what is the difference between a Vertical Eight and a Horizontal Eight?

Vertical Eight the one loop is above the other, Horizontal loops follow on the same level.

10.	Which way up is the aircraft at the top of a loop?	inverted

11.	What are the two parts to a Roll off the top?	……half loop, half slow roll	………………….

12.	Which manoeuvre starts with a slow roll and finishes with a loop?	Half roll and pull through.

13.	In an Upward roll is the horizontal or the vertical axis used?		vertical

14.	Which manoeuvre has either four point or eight point rolls?		hesitation rolls ………….

15.	Which gives a higher gliding speed, right way up or inverted flight?	…inverted

16.	One manoeuvre involves a steep turn rolling through the inverted position.  Which is this?

Derry turn	……………………………………………

17.	Why is a line feature, such as a railway line, useful in a barrel roll?

to keep the roll centre line going straight

You are an aerobatic pilot.  What sequence of manoeuvres might you use for a display?

7 FORMATION FLYING

Have you ever watched the Red Arrows or any other team doing formation flying?	………………..

Think of the skill and precision flying needed.

If you just put a group of experienced pilots with suitable aircraft together and told them to go and fly in formation would they be able to do it if they had not met before?  Give a reason for your answer.

No.  Lack of team work, trust and practice.

What is the minimum number for formation flying?	………2……………………………………….

Together the formation flyers make a    team… which must have a …leader…………………

What is the term for the person responsible for briefing and safety?	…number 1……………..

In a formation how many pilots will use instruments? …1………….  they are …number 1……….

What must all the others do?	…follow……………………………………………………………….

What two categories of formation flying are used by the military?	close, tactical

In a display which category is the Red Arrows using?	close………………………………

How many pilots can communicate with ground? 	…1………….  they are …… number 1….

Below are 5 boxes.  Use the manual to copy the 5 standard formations, naming each one.

……Line Abreast

…Echelon….

…Vic……..

1

2

3

1

2

3

1

2

3

Line Astern

…Box.

Formation techniques

Speed:  What speed were the Red Arrows doing in the video?	……400 mph………………..

Distance: What distance was between the Red Arrows?	……6 – 11ft………………………………

Measure that distance in the classroom.  Does it seem far or near for an aircraft? ……near……….

Size:	What appears to happen to the size of an object as it gets closer? …larger……………..

Joining:  Look at the joining diagram.	Who goes first?	… number 1.

What do you notice about the joining turn of each aircraft from 2 – 4? smaller radius, shorter turn……….

What is a way of making sure that you always keep your place in a formation?

two references lined up perfectly eg: keeping the aircraft in front at a certain position eg; nose onto wingtip

If you are flying correctly will the other aircraft appear to be moving compared with you? no………….

For station changes which parts do you use for the following changes?

Longitudinal:	Throttle……  and ………elevator……………….

Lateral:	aileron………......   Vertical: …elevator…………………..

For change of speed a Clean Aircraft:

…accelerates…….  quickly;  	…………decelerates………   slowly.

What problem does a jet aircraft have?	…does not accelerate quickly from low speed……………

Formation on the ground

What do cadets do on the ground that is similar to formation flying? …parade drill ……................

Practice doing the formations you have drawn, but on the ground.  Think how you would do it.

Choose one you have practiced, describe what you have done and what problems there were:

8 EMERGENCY PROCEDURES

Considering flying an aircraft, think of 3 possible emergencies and how you might deal with them:

Emergency					Dealing with it

Look at the Feedback newsletter on the website by Emergencies for examples.

…………………………………….		…………………………………………………………………….

…………………………………….		……………………………………………………………………..

This section will deal with Emergency Procedures using the radio and radar.

What are the two degrees of emergency?	distress…….  and …urgency………..

Circle in red the one of the highest level.

It is important to communicate with others when you have an emergency. Knowing frequencies is essential.

Give the two types and the frequencies:  Radio Telephony  R/T  243.000 MHz  military 121.5 civ…….

…Wireless Telegraphy,  W/T  500KHz  (HF)

What is the Distress call?			R/T Mayday, mayday, mayday  and W/T SOS SOS SOS

Give the Morse Code for the Distress call:	…	…---…  …---…  …---…

What is the Urgency call?				R/T   PAN PAN PAN  and W/T  XXX  XXX  XXX.

Give the Morse Code for the Urgency call:	… 	 -..-  -..- -..-   -..- -..- -..-   -..- -..- -..-

Your aircraft is G ABCD, give this in Morse:		--. .- -… -.-. -..

9 more points need to be given in your emergency call, list them in this table:

| Position | Time | Heading | Airspeed | Altitude | Type of aircraft |
| --- | --- | --- | --- | --- | --- |
| Nature of emergency |  | Intentions of captain |  | Endurance remaining |  |

You are flying along in G ABCD and there is an emergency.  Decide on your aircraft, the emergency, where you are etc and work out an emergency call.   Put it into this table.  You will tell this to the group.

| Mayday, Mayday, Mayday | G ABCD |
| --- | --- |
| West abeam Metfield gliding site | At 15.30 |
| Routing 60deg | At 100kts |
| Altitude 3000’ on QNH 1019 | In a PA28 Warrior |
| Oil spraying over the cowling, engine uncertain | Diverting to Beccles |
| Endurance half an hour |  |

Fixer Service: 	S econdary.   S urveillance.  R adar………

Fill in the above heading.

What does Radar stand for?				Radio Detection And Ranging

Which machine in the aircraft is using radar?	Transponder……………………….

What is the purpose of the radar?			To be detected, tracked, found

Normal radar code is:					7000

Three emergency radar codes are:

Terrorist: ……7500		………   Radio failure: …7600………… Other problems: 7700……

Final Transmission: this is when you have to abandon your aircraft.

What do you do with your transmission switch?	leave it on	………………….

What else could you use?				PLB, Personal Locator Beacon

UHF Emergency Fixer Service

What is UHF?	…Ultra High Frequency	  What is the emergency frequency?	……243.00 MHz..

To where is it relayed?	ATCC

What is the problem in Scotland?	Mountains, so only accurate 5000 – 8500 ft …………

S earch  A nd.  R escue..  S atellite..  A ided  Tracking

Fill in the above heading.

How is it activated?			by any emergency frequency calls

What should you do if the emergency is cancelled or you hit the emergency by mistake?

make sure all calls on all frequencies are cancelled

If you see or hear another aircraft having problems what should you do?  That is First Aid.

1. 	A Visual emergency:

keep aircraft in site, contact ground controller, guide others, follow instructions from ground

2.	A radio emergency:

take bearings, plot position, listen on frequency, call, contact ground control, proceed to emergency location.

Emergency Organisations

Give the full names for ATCC:	Air Traffic Control Centre.

and ATCRU:				Air Traffic Control Radar Unit

Triangular patterns

Why might an aircraft be flying in a triangular pattern?	radio problems

What type of triangle is this?					equilateral

How many degrees must the aircraft turn at each corner?	120…………………………………

Draw the two triangles.

Show the direction of flight on each one.

Flight direction		left

Reason	transmitter and receiver failed

Label the meaning for each one.

Flight direction		right

Reason	transmitter failed

Search and Rescue Services

Search and Rescue Services are important for more than aircraft.

Think of other ways they could be used:

boats at sea, walkers in the Highlands, …………………….

What does RCC stand for:		Resource Co-ordination Centre

What facilities are included:

SAR helicopters, lifeboats, long range maritime patrol aircraft, mountain rescue teams and police and ambulance services.

Now go through the whole workbook and check that everything is completed, including the abbreviations on the next page.

Abbreviations  Following is a list of abbreviations that you should know.  Fill in their meanings:

| AAR | Air to Air Refuelling |
| --- | --- |
| ATC | Air Traffic Control |
| ASP | Aircraft Servicing Platform |
| ATCC | Air Traffic Control Centre |
| ATCRU | Air Traffic Control Radar Unit |
| AVCAT | Aviation Turbine Fuel – navy |
| AVGAS | Aviation gasoline, similar to petrol |
| AVTAG | Aviation Widecut Gasoline |
| AVTUR | Aviation Turbine, kerosene |
| BCU | Bird Control Unit |
| CDTC | Combat Development Test Centre ? |
| C of G | Centre of Gravity |
| DFTS | Defence Fixed Telecommunications Service |
| FIR | Flight Information Region |
| FOD | Foreign Object Damage |
| GCA | Ground Controlled Approach |
| HASELL | Height, Airframe, Security, Engine, Location, Lookout – for aerobatics |
| HUD | Head Up Display |
| IAS | Indicated Air Speed |
| IFF/SIF | Identification Friend or Foe, / Selective Identification Feature |
| ILS | Instrument Landing System |
| IMC | Instrument Meteorological conditions |
| MDS | Maintenance Data System |
| MOD | Ministry of Defence |
| NATO | North Atlantic Treaty Organisation |
| NBC | Nuclear Biological Chemical – hostile environments |
| NDT | Non-destructive testing, eg X-rays to reveal fatigue cracks |
| PAR | Precision Approach Radar |
| QFE | Air pressure at Field Elevation |
| QNH | Air pressure at sea level |
| RADAR | Radio Detection and Ranging |
| RCC | Resource Coordination Centre |
| RT or R/T | Radio Telephony/Transmission |
| SARSAT | Search and Rescue Satellite Aided Tracking |
| SOAP | Spectrum Oil Analyzed Particles, part of continuous testing for condition based maintenance |
| SSR | Secondary Surveillance Radar |
| UHF | Ultra High Frequency |
| VHF | Very High Frequency |
| VMC | Visual Meteorological Conditions |
| W/T | Wireless Telephony |

Marshalling, descriptions for cut and paste